GENERAL

Scope of the Instructions for Use:

These instructions for use are valid for the following instruments collectively referred to as SS WHITE DENTAL Burs: Clinical Carbides: Operative burs, Great White® Series, trimming and finishing burs, oral surgery burs, fissurotomy burs; Laboratory burs.

# Intended use:

- SS WHITE DENTAL Burs are intended to fit into a dental handpiece, which provides the rotation, allowing the user to cut hard structures in the mouth, e.g., teeth or bone. SS WHITE DENTAL Burs can also be used to cut hard metals, plastics, porcelains and similar materials.

# Indications for use:

- This product is to be used on the instruction of, or by a licensed dentist or other licensed practitioner. Carbide burs are single use invasive devices to be used sterile, indicated for treatment of patients with dental disease such as caries, or hard tissue damage.

- SS WHITE DENTAL Burs are available in Sterile and Non-Sterile models under various trade names with numerous head diameters and working lengths.

# Contra-indications:

- Do not use on individuals with a known allergic sensitivity to this metal.

# Device Description:

- The SS WHITE DENTAL Bur family includes Clinical Carbide Burs and Laboratory Carbide Burs. They are designed to fit into a dental motor and handpiece system which provides the rotation, allowing the user to safely shape and debride tooth and restorative structures.

- SS WHITE DENTAL Burs are available in Sterile and Non-Sterile models under various trade names with numerous head diameters and working lengths. The devices provided as non-sterile can be sterilized using steam sterilization in an autoclave.

# Patient Population:

- SS WHITE DENTAL Burs are general dental instruments intended for use during dental procedures to enable the user to safely shape and debride tooth and restorative structures in the general patient population.

# Intended User:

- These devices are not to be used by unqualified personnel. For use by or on the instruction of dental professionals only.

# Expected Clinical Benefits:

- Successful cutting of hard structures such as teeth or bone, hard metals, plastics, porcelain and similar materials, in the mouth.

# Composition:

- A CARBIDE dental bur is a rotary cutting device made of stainless steel, the working end of which is made from tungsten carbide, and which is designed to fit into a dental handpiece. Alternatively, the device may be constructed wholly from tungsten carbide.

# Instructions for use:

Technical Knowledge, Experience, Education or Training of Intended User:

For use by or on the instruction of dental professionals only.

# General instructions:

- Prior to use, sterilize the burs supplied as non-sterile in accordance with the validated procedures provided below.

- Select the correct handpiece matching the shank of the bur being used.

- Release the collet and fully insert the bur into the handpiece.

- Ensure that the device is secure before use.

- Do not force a bur into the handpiece. In case of difficult access, check both handpiece turbine and bur and refer to handpiece instructions for troubleshooting

- Always refer to the product packaging for the Maximum RPM. Use of the bur beyond the RPM range may cause the bur to break and result in patient or user harm.

# WARNINGS:

  - Although SS White products are single use devices, risk of reuse can include:

    - Cross contamination

    - Failure to due to fatigue

    - May lead to decreased cutting efficiency which could result in device failure or generation of undesirable heat and cause patient discomfort, tooth or tissue necrosis, or patient burns.

- The device is to be used by or on the instruction of dental professionals only.

- Attention should be paid to the speed of work (RPM)

  - Always refer to the product packaging for the Maximum RPM. Use of the bur beyond the RPM range may cause the bur to break and result in patient or user harm.

  - Operating a bur with too high of an RPM may generate undesirable heat and cause patient discomfort, tooth or tissue necrosis, or patient burns.

- Proper irrigation is required while using the device. Inadequate use of irrigation may generate undesirable heat and cause patient discomfort, tooth or tissue necrosis, or patient burns.

- Do not apply excessive pressure on the bur as this could cause undesirable heat or may cause the bur to fail and cause patient or user injury.

- CARBIDE Burs must be steam sterilized prior to the first use to prevent the risk of infection or cross-contamination.

- CARBIDE Burs labeled and sold as “Sterile” require no further action prior to first use

- If the packaging for “Sterile” labeled devices is opened or damaged or is past its expiration date, the device must be thoroughly cleaned and steam sterilized prior to use to prevent the risk of infection or cross-contamination.

- Do not use chemical or dry heat to sterilize SS WHITE DENTAL Burs, as these processes have not been validated for use. Use of these processes may be corrosive to the device and could result in premature device failure.

- Use a rubber dental dam while using CARBIDE Burs to avoid possible aspiration or swallowing of the device.

- Always wear gloves when handling contaminated instruments to avoid possible infection/cross-contamination.

- Eye protection must be worn to protect against eject particles which could cause user injury.

- Surgical masks must be worn to avoid inhalation of any aerosol or dust generated that could cause possible infection/cross-contamination.

- Carefully read package labels to ensure use of the appropriate device. Failure to do so may cause procedural delays or patient or user injury.

- Failure to follow these instructions may cause the following: preparation site damage, injury to the patient or user, or possible aspiration or swallowing of the CARBIDE Burs.

# PRECAUTIONS:

- Carefully read package labels and instructions to ensure safe use of the appropriate device.

- This product may contain Nickel, a chemical known to the state of California to cause cancer, birth defects or other reproductive harm.

- Always inspect the burs before use

- Use a rubber dental dam while using burs to avoid possible aspiration or swallowing of debris.

- Use of worn-out or dull burs could cause undesirable heat or may cause the device to fail.

- Use of bent burs could cause vibration that may cause patient discomfort or damage to the preparation site.

- Move the bur continuously when in use to avoid localized heating and/or damage to the bur. Undesirable heat generation can cause patient discomfort, tooth or tissue necrosis, or patient burns.

- Avoid removing the bur at too sharp an angle to avoid leverage and breakage which could cause patient or user injury.

- Maintain handpieces in good working condition to ensure maximum effectiveness of the device. Failure to properly maintain handpieces may lead to procedural delays or injury of the patient or user, aspiration or swallowing of the device or damage to the preparation site due to vibration of a worn chuck or turbine.

- Ensure the bur is fully seated and securely gripped in the handpiece collet prior to use. Failure to do so may cause the device to “walk out” of the handpiece and may lead to injury of the patient or user or aspiration or swallowing of the device.

- Never force a bur into a handpiece as this could cause damage to the handpiece collet which could result in procedural delays.

- SS WHITE DENTAL Bur Blocks used to hold the devices for storage and steam sterilization are not intended to maintain sterility of the device. Burs should be stored in the sterilization pouch to prevent cross-contamination.

- Do not force CARBIDE Burs into Bur Blocks as this could cause damage to the bur or cause it to become lodged in the Bur Block.

# PERSONAL PROTECTION EQUIPMENT:

  - Always wear STERILE gloves when handling STERILE and/or contaminated instruments.

  - Eye protection must be worn to protect against potentially ejected particles.

  - Surgical masks must be worn to avoid inhalation of any aerosol or dust generated.

# ADVERSE EVENTS/COMPLICATION/UNDESIRABLE SIDE-EFFECTS:

Report any serious incident occurring in relation to the device to the manufacturer and the competent authority of the Member State where the user/patient is established.

A serious incident is any incident that directly or indirectly leads to, or might have led to, any of the following:

  - Patient death, or death of user or other person

  - Serious deterioration (permanent or temporary) of patient or users or other person’s health

  - A serious public health threat

For more information on adverse events involving specific dental procedures or devices, see the Instructions for Use of the for that product. General dental procedures have possible adverse events to the patient including:

  - Procedural delay due to missing, damaged, or worn instruments

  - Hard or soft tissue injury and/or additional tooth removal due to blunt, damaged, or incorrectly used instruments

  - Allergic reactions due to materials in contact with the patient

  - Infection due to improper instrument processing

  - Soft tissue damage due to sharp instruments

  - Neurological damage from sharp instruments or procedural trauma

  - Device fracture/breakage can lead to complications which can include pain, bleeding, soft tissue damage, hard tissue damage, and/or infection.

  - Use of product when package is damaged or opened  can lead to infection.

  - Complications associated generally with dental procedures can include pain, bleeding, soft tissue damage, hard tissue damage, and/or infection.

# DISPOSAL OF USED PRODUCT:

  - After use, products should be disposed appropriately by user as biohazard material per local governing ordinances.

# STORAGE CONDITIONS:

  - Carbide burs should be stored in an ambient operatory temperature/environment, away from sunlight and corrosive chemicals, in a clean, dry location.

  - The Carbide Burs should be stored in the sterilization pouch (or instrument block) until required.

# STERILIZATION INSTRUCTIONS:

Refer to IFU001 Storage, Cleaning and Sterilization IFU.

  - If the package is not labelled Sterile, then the burs must be steam sterilized* before use.

  - Non-sterile carbide burs are to be sterilized in accordance with the procedures provided in IFU001 prior to use.

  - Sterilized carbide burs MUST be stored in their sealed, sterile barrier pouch to maintain sterility UNTIL USE.

  - Sterile Carbide Burs do not require sterilization prior to use UNLESS the sterile barrier packaging is damaged.

If the sterile barrier packaging is damaged, the burs must be inspected, cleaned and sterilized in accordance with the validated procedures provided in IFU001 prior to use.

  - Carbide Burs are single use only and may be available pre-sterilized via radiation (in sterile barrier packaging) or non-sterile as identified on the packaging.

  - NOTE: Blocks used to hold the devices for storage and steam sterilization are not intended to maintain sterility of the device.

*Please see sterilization instructions included in IFU001.

Glossary of Symbols

| Graphic | Title |  | Graphic | Title |
| --- | --- | --- | --- | --- |
|  | Manufacturer |  |  | Caution |
|  | Consult instructions for use |  |  | Date of Manufacture |
|  | Batch Code or lot number |  |  | European Conformity |
|  | Catalog Number |  |  | Authorized representative in the European Union |
|  | Use by Date |  |  | Recyclable |
|  | Do not re-use |  |  | Do not use if package is damaged |
|  | Sterilized using irradiation |  |  | Importer |
|  | Translation |  |  | Medical Device |
|  | Autoclave at specific temperature |  |  | Contents Symbol |
|  | Unique Device Identifier |  |  | Model Number |
|  | European Conformity |  |  | Indicates the maximum and minimum temperature limits at which the item shall be stored, transported or used. |
|  | Revolutions Per Minute |  | only | Caution: Federal Law restricts this device to sale by or on the order of a “Dentist/Physician |

1145 Towbin Ave • Lakewood, NJ 08701 732-905-1100 | 800-535-2877

www.sswhitedental.com

SS White® Dental is a registered trademark of SS White Burs, Inc. - All rights reserved.